Lab #1

Group number : 5

Group members : Aditya Ray, Krutanjay

- Prelab

  - What flag for ls displays all files, even hidden ones?
⇒ -al

  - How do you move to the parent directory of the current one?
⇒ cd ..

  - What is the difference between the mv and cp commands?
⇒ mv moves a file

- cp copies a file

  - What does the -h flag of ls do? Hint: use man to find out.
⇒ -h flag is used for printing the file and folder sizes in human readable format instead of number of bytes (eg: 2K,128M, etc)

  - In a git repository, what command displays whether there are any local changes and what they are?
⇒ git status

  - In a git repository, what does git pull do?
⇒ It synchronises the remote and local repositories

- Checkoffs

  - Verify that the VS Code and command-line mechanisms are working. Explain how the timing of the blinking of the LED is controlled.
⇒ The timing of the LED is controlled by a timer with a specified period and offset.

- timer t(offset, period)

- This timer then triggers a reaction that flips the state of the LED at the specified period and prints the state.

  - Show your modified LF program. Explain how this use of timers is different from the sleep function used in the C code blink.c.
⇒

- ToolsLEDSolution.lf

- target C {

- platform: {

- name: "rp2040",

- board: "pololu_3pi_2040_robot"

- },

- single-threaded: true

- }

- preamble {=

- #include <stdio.h>

- #include <pico/stdlib.h>

- #include <hardware/gpio.h>

- =}

- main reactor {

- timer ton(0, 200ms)

- timer toff(100ms, 200ms)

- reaction(startup) {=

- gpio_init(PICO_DEFAULT_LED_PIN);

- gpio_set_dir(PICO_DEFAULT_LED_PIN, GPIO_OUT);

- =}

- reaction led_on(ton) {=

- printf("LED State: %b\n", true);

- gpio_put(PICO_DEFAULT_LED_PIN, true);

- =}

- reaction led_off(toff) {=

- printf("LED State: %b\n", false);

- gpio_put(PICO_DEFAULT_LED_PIN, false);

- =}

- }

- LED.lf

- target C {

- platform: {

- name: "rp2040",

- board: "pololu_3pi_2040_robot"

- },

- single-threaded: true

- }

- preamble {=

- #include <hardware/gpio.h>

- =}

- reactor LED {

- input set:bool;

- reaction(startup) {=

- gpio_init(PICO_DEFAULT_LED_PIN);

- gpio_set_dir(PICO_DEFAULT_LED_PIN, GPIO_OUT);

- =}

- reaction(set) {=

- printf("LED State: %b\n", set->value);

- gpio_put(PICO_DEFAULT_LED_PIN, set->value);

- =}

- }

- ToolsBlinkSolution.lf

- target C {

- platform: {

- name: "rp2040",

- board: "pololu_3pi_2040_robot"

- },

- single-threaded: true

- }

- import LED from "lib/LED.lf"

- reactor led_on{

- timer t(0, 250ms)

- output set: bool

- reaction led_on(t)->set {=

- lf_set(set, !set->value);

- =}

- }

- main reactor {

- led = new LED()

- led_driver = new led_on()

- led_driver.set -> led.set

- }

- sleep() would stop the local time instead using timer it creates events in the logical time instead of stopping it altogether.

  - Show ON-OFF output.
⇒ Shown in video. OR Shown in person.

  - Show and explain how your program works.
⇒

- Postlab

  - What format specifier(s) for printf allows the printing of floats (there may be several)?
⇒ We use %f for floats but if we want upto a specific decimal we can use %0.2f got two decimal places or %0.4f for 4 decimal places.

  - When might printf statements be the best tool for debugging?
⇒ it is best when there are sequencial events and we have access to stdout stream.

  - What other tools might be useful for debugging embedded software (note that using an interactive debugger like gdb with the robot or Pico board is possible but requires extra hardware)?
⇒ We can use GPIO pins to check timings. Core dumps are useful for debugging crashes. Assert statements are also useful to check for conditions at different points of the code. Store logs with time stamps in a spetial memory location and then reteive it after execution has finished.

  - What does the volatile keyword mean in C and when might you use it?
⇒ Volatile is a storage class that specifes the compiler not to optimize the variable. This is used inside ISRs.

  - What were your takeaways from the lab? What did you learn during the lab? Did any results in the lab surprise you?
⇒